Project Title

Basic Traffic Light Control System (Tinkercad Simulation)

Project Objective

To simulate a real-world traffic light system using an Arduino Uno that controls Red, Yellow, and Green LEDs in a timed sequence, similar to an actual road intersection.

This project helps students understand:

- Digital output control

- Timing using delay()

- Basic electronics and microcontroller programming

- Real-world system logic

Components Required (Tinkercad)

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| Red LED | 1 |
| Yellow LED | 1 |
| Green LED | 1 |
| Resistors (220Ω) | 3 |
| Breadboard | 1 |
| Jumper wires | As needed |

Step 1: Open Tinkercad

- Go to tinkercad.com

- Login

- Click Circuits

- Click Create new Circuit

Step 2: Place Components

Add the following to the workspace:

- Arduino Uno

- Breadboard

- 3 LEDs (Red, Yellow, Green)

- 3 Resistors (220Ω)

Step 3: Circuit Connections

LED Connections

| LED Color | Arduino Pin | Connection |
| --- | --- | --- |
| Red | Pin 13 | Through 220Ω resistor |
| Yellow | Pin 12 | Through 220Ω resistor |
| Green | Pin 11 | Through 220Ω resistor |

Wiring Rules

- Long leg (Anode) → Arduino pin via resistor

- Short leg (Cathode) → GND rail on breadboard

- Connect breadboard GND rail to Arduino GND

Step 4: Logic of Traffic Light

| State | Red | Yellow | Green | Time |
| --- | --- | --- | --- | --- |
| Stop | ON | OFF | OFF | 5 sec |
| Ready | OFF | ON | OFF | 2 sec |
| Go | OFF | OFF | ON | 5 sec |

This cycle repeats continuously.

Step 5: Arduino Code (Copy to Tinkercad)

// Traffic Light System

// Beginner IoT Project

int redLED = 13;

int yellowLED = 12;

int greenLED = 11;

void setup() {

pinMode(redLED, OUTPUT);

pinMode(yellowLED, OUTPUT);

pinMode(greenLED, OUTPUT);

}

void loop() {

// RED light ON (STOP)

digitalWrite(redLED, HIGH);

digitalWrite(yellowLED, LOW);

digitalWrite(greenLED, LOW);

delay(5000);

// YELLOW light ON (READY)

digitalWrite(redLED, LOW);

digitalWrite(yellowLED, HIGH);

digitalWrite(greenLED, LOW);

delay(2000);

// GREEN light ON (GO)

digitalWrite(redLED, LOW);

digitalWrite(yellowLED, LOW);

digitalWrite(greenLED, HIGH);

delay(5000);

}

Step 6: Run the Simulation

- Click Start Simulation

- Observe LEDs switching like a real traffic light

Step 7: Explanation for Students

- pinMode() tells Arduino which pins control LEDs

- digitalWrite(HIGH) turns LED ON

- digitalWrite(LOW) turns LED OFF

- delay() controls timing (milliseconds)